Software Version	3.16.10

Experiment File Path:	C:\Users\Public\Documents\Experiments\20261020-mgig-samuel-res-learning\Experiment1.xpt

Protocol File Path:	C:\Users\Public\Documents\Protocols\resazurin_12well.prt

Plate Number	Plate 10

Date	10/20/2026

Time	11:03:39 AM

Reader Type:	Synergy HTX

Reader Serial Number:	25030704

Reading Type	Reader

Procedure Details

Plate Type	12 WELL PLATE

Eject plate on completion

Plate Out/In

Read	Fluorescence Endpoint

Full Plate

Filter Set 1

Excitation: 528/20,  Emission: 590/20

Optics: Top,  Gain: 35

Light Source: Tungsten,  Standard Dynamic Range

Read Speed: Normal,  Delay: 100 msec,  Measurements/Data Point: 10

Read Height: 4.5 mm

Actual Temperature:	23.7

Results

1	2	3	4

A	267	3808	2832	3758	528/20,590/20

B	2517	2859	4030	3695	528/20,590/20

C	3816	2028	3558	3035	528/20,590/20